A Project Report on

“SMART TRAFFIC MANAGEMENT TO REDUCE

AMBULANCE DELAY”

Submitted in partial fulfilment of the requirements for the award of the degree

BACHELOR OF TECHNOLOGY

in

### ELECTRONICS AND COMMUNICATION ENGINEERING

by

# RAVI KUMAR (ENG21EC0095)

# UDAY REDDY (ENG21EC133)

# SYED OMAR (ENG21EC0127)

# MAHENDRA B (ENG20EC0048)

Under the guidance of

DR.SUPRAJA EDURU

Assistant professor

DEPARTMENT OF ECE,

SCHOOL OF ENGINEERING, DSU

### Department of Electronics and Communication Engineering,

### Dayananda Sagar University, Bengaluru 560114

#### Academic Batch:2021	Year:2025

ABSTRACT

The rapid increase in urban vehicular traffic has escalated congestion levels, especially during peak hours, severely impeding emergency response times. Ambulances, critical for saving lives in emergencies, often face delays due to inefficient traditional traffic management systems. This project presents a novel IoT-based Smart Traffic Management System designed to mitigate these issues by prioritizing ambulances in traffic. The system leverages advanced technologies like image processing, IoT, and real-time data analytics to dynamically adjust traffic signals. Real-time video feeds from cameras and audio inputs from microphones are processed to detect ambulances based on their visual and auditory signatures. An intelligent algorithm, using techniques like the Canny Edge Detector, identifies traffic density and emergency vehicles. Once an ambulance is detected, the system overrides standard traffic light operations to provide a green signal along the ambulance's path, ensuring rapid and uninterrupted movement through congested areas.

This project is motivated by the pressing need for smarter urban traffic solutions. Increasing traffic congestion, delays in emergency medical assistance, and the limitations of traditional static traffic systems highlight the importance of adaptive and intelligent systems. By utilizing IoT devices, sensors, and machine learning algorithms, the system not only reduces ambulance travel times but also improves overall traffic flow efficiency. The methodology involves data collection from webcams, sensors, and microphones to -analyze traffic density and detect emergency vehicles. Logical operations and decision-making algorithms manage the traffic light states dynamically, switching between normal and ambulance-prioritized modes. For instance, the system follows a standard traffic light sequence during regular operations but immediately transitions to prioritize ambulances when detected. This decision-making is implemented using components like Raspberry Pi, Arduino boards, and audio detectors. Simulations were conducted using MATLAB and Simulink to validate the system’s performance. The code dynamically adjusts signal states, ensuring a green light for ambulances while maintaining logical transitions for other traffic. Hardware components such as Raspberry Pi cameras, Arduino Mega 2560, and microphones are integral to the real-time implementation.

Table of Contents:

- Introduction to Technology

- Literature Survey- (Related work done in last 5 years, minimum 15 research papers)

- Problem Statement

- Methodology

- Details of the components/simulation tools to be used

- Deliverables

- Expected Output based on understanding from literature survey

- Timelines

- Contribution of each member

- References

- Publications

Introduction to Technology

Long commutes and large delays in vehicle flow at crossings are the results of the expanding number of four-wheeled cars on metropolitan streets, which has greatly worsened traffic congestion, especially during peak hours.  It becomes a very hazardous scenario when ambulances are delayed during crises due to the limitations of traditional traffic management systems that are not adaptable enough to change in real time. This problem is addressed by the proposed system, which introduces an intelligent, Internet of Things-based smart traffic management solution that prioritizes ambulances by dynamically changing traffic lights in response to real-time inputs.   In addition to advanced technologies like image and audio processing, the device uses real-time video feeds and auditory data captured by strategically placed microphones to detect emergency vehicles. Yolo and other machine learning algorithms are used to detect ambulances and measure traffic density, allowing for prompt and precise decision-making.  The device ensures quick and continuous mobility by overriding conventional traffic light logic to give an ambulance a green signal along its path once it is spotted.

Furthermore, under typical circumstances, the system works effectively by tracking vehicle density and adjusting signal timings appropriately to provide the least amount of disruption to normal traffic.  A scalable and reasonably priced hardware configuration is created by combining microcontrollers, such as the Arduino nano CH340, with sensors and real-time control software.  MATLAB and Simulink simulations verify that the technology can drastically cut ambulance delays while preserving smooth traffic flow.This intelligent infrastructure improves air quality, reduces fuel consumption, and eases traffic congestion, all of which contribute to sustainable urban development and emergency response efficiency.  In the direction of safer, smarter, and more effective urban transportation networks, this technology is a revolutionary move that will integrate with future smart city efforts and facilitate data-driven decision-making.It was suggested that lanes be automatically cleared for ambulances using an RFID-based system.  In dense urban areas, roadside readers can identify RFID tags on ambulances, which causes traffic lights to turn green and offer a clear path, reducing delays and speeding up emergency response times [1].  It was suggested to employ traffic flow models to forecast and control congestion in a smart traffic system.  In metropolitan locations, the system analyzes vehicle density and flow rates to dynamically improve traffic signal timings, lowering waiting times and increasing overall traffic efficiency [2].

Additionally, a smart traffic management system based on the Internet of Things was created that uses sensors to gather real-time traffic data and adjusts traffic lights accordingly.  The system seeks to increase road safety, reduce traffic jams, and expedite the clearance of emergency vehicles such asemergency vehicles [3].  AI algorithms are used to assess traffic trends and forecast congestion in a deep learning- based smart traffic management system for smart cities.  Additionally, by enabling proactive traffic control, the system enhances road safety, minimizes delays, and maximizes traffic flow across the city [4].

A smart traffic management system based on the Internet of Things (IoT) that uses connected sensors and gadgets to monitor traffic conditions.  The goal of the system is to create safer and more effective urban transportation networks by processing real-time data to dynamically control traffic lights and provide precedence to emergency vehicles.[5] This study examined device-to-device (D2D) interactions in smart traffic systems based on the Internet of Things.It emphasizes how decentralized real-time information transmission promotes decision-making in traffic management, optimizes traffic flow, and improves data sharing when vehicles and infrastructure communicate directly [6]

Literature Survey

| S.No | Author | Year | Objective | Methodology | Key Findings | Limitations |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | P Phani Kumar, Judy Simon, K Durga Devi, M Aarthi Elaveini, and N Kapileswar | 2023 | Research Purpose | RFID "Wireless Sensor Network" | This study proposes an RF sensor- based system with a priority-based MAC to enhance real-time traffic management by rapidly transmitting emergency vehicle data, enabling quicker response and smoother passage in traffic. | A limitation of this system is its dependency on RF sensor range and signal interference, which may affect communication reliability in dense traffic or complex urban environments. |
| 2. | Rajeshwari Sundar, Santhoshs Hebbar, and Varaprasad Golla | 2015 | Research Purpose | Traffic Control System for Congestion Control, Ambulance Clearance, and Stolen Vehicle Detection | This intelligent traffic control system uses RFID and ZigBee to prioritize emergency vehicles, adjust green light timing, and detect stolen cars, showing effective results in lab tests. | The system's reliance on RFID and ZigBee may limit effectiveness due to potential signal interference and range constraints. |
| 3. | Rajib Kumar Kar, A.S.S. Murugan*, and Manohar Gangikunta | 2024 | Research Purpose | Arduino-based Intelligent Emergency Ambulance and Traffic Management System | The Arduino-based system adjusts traffic signals based on density, reducing congestion and improving emergency vehicle flow. | A limitation of this system is its reliance on traffic density sensors, which may not always accurately capture real-time traffic conditions in complex or fluctuating traffic patterns. |
| 4. | Koushik Mandal, Arindam Sen, Abhijnan Chak raborty and Siuli Roy | 2011 | Research Purpose | Road traffic congestion monitoring and measurement using active RFID and GSM technology. | Using Active RFI D they can monitor and read long range object with low power Consumpti on. | Use of Active RFID in real time Scenario is Bit more difficult because of it limited lifetime and Expensive. |
| 5. | SHRUTHI, Κ. R. and VINODHA. К. | 2013 | Research Purpose | Priority based traffic lights controller using WSNs. | They Use Fuzzy logic Here. Fuzzy logic perform better than fixed time controller. | Fuzzy logic use Human reaso ning and its need expert guidance to built and update. |
| 6. | Ms. Madhavi N. Shrikhande Prof. Dipali K.Shen de | 2014 | Research Purpose | Embedded web technology in traffic monitori ng system | Using This Method we can monitoring and Control via web browser. | This method is costly, inefficie nt, and troublesome for the system to update |
| 7. | V. V. Dabahde, Dr. R. V. Kshirsagar | 2015 | Research Purpose | FPGA- Based Intelligent Traffic Light Controller System Design | FPGA is low cost and able to reuse and reconfigure e asily | It has High complexity and less Performa nce. |
| 8. | Prof. Deepali Ahir, Saurabh Bharade, Pradnya Botre, Sayali Nagane, Mihir Shah. | 2018 | Research Purpose | The proposed system clears the traffic congestion by turning all the red lights to green on the path of the ambulance. | The system consists of an android application which registers the ambulance on its network In case of emergency situation, if the ambulance ha on its way, the application sends an emergency mmand the traffic signal server | Implementaion of such project is diffcult having cost, privacy concerns and RFID range mind The technical issues can be susceptible to malfunctions and glitches, which can lead to incorrect traffic management decisions. |
| 9. | Ankush, G Sai Pavan, Harihran M, Jagadish Naik, Kiran S M | 2019 | Research Purpose | The system will detect vehicles through images instead of using electronic sensors. | A camera installed alongside the traffic light will capture image sequences. The image sequence will be analyzed using digital image processing techniques. | It can have many limitations such Inaccura data, detecting vehicles, optical flow method, frame difference.vehicle tracking Tracking vehicles can be challenging due to abrupt vehicle motion, changing appearance patterns, and vehicle-to-vehicle interactions |
| 10. | R.Krishnamoorthy Sethu Manickam | 2018 | Research Purpose | Vehicle Detection shall be carried out in DIP by applying a simple kernel-based Edge Detection | Subsequent process carried out novel Traffic Scheduling Algorithm through the help of a hybrid Round Robin having a dynamic time slice obtained by using Longest Remaining Job First periodically update the traffic signal lights to relax the traffic. Instead of turning on the green light for fixed amount of time, the duration will be managed dynamically based the amou of traffic in each road. | It can have several limitations including vehicle detection, vehicle tracking.data volume, real time analysis,congestion detection, placement mera |

Problem Statement

- Urban traffic congestion delays ambulances, impacting response times and efficiency. Current signals lack real-time adaptability. An IoT system is needed to monitor traffic, prioritize emergency vehicles, and adjust signals dynamically to reduce delays.

Methodology (block diagram)

Emergency vehicle is detected

Image signal processing and emergency vehicle detection

Emergency vehicle is  detected

Capture the image of emergency vehicle at a certain distance from the traffic signal

Emergency vehicle detection using YOLO V8 model

Change the traffic light signal to green

Continue with the operation of automatic traffic signaling system based on traffic density

Emergency vehicle not detected

Capture the image of the emergency vehicle at the traffic signal

Emergency vehicle not detected

Continue the traffic light signal to green

Methodology

1. Capture the image of emergency vehicle at a certain distance from the traffic signal

The system captures images of oncoming vehicles at a predefined distance from the traffic signal using a camera.

2. Emergency vehicle detection using YOLO V8 model

The captured image is processed using the YOLO V8 deep learning model to identify whether an emergency vehicle (e.g., ambulance) is present.

3. Emergency vehicle is detected

If the YOLO model detects an emergency vehicle, the system proceeds to give it priority.

4. Change the traffic light signal to green

The traffic light is immediately turned green to allow the emergency vehicle to pass through without stopping.

5. Capture the image of the emergency vehicle at the traffic signal

A second image is taken as the emergency vehicle reaches the signal, for further confirmation.

6. Image signal processing and emergency vehicle detection

This second image is analyzed again to verify whether the emergency vehicle is still present at the intersection.

7. Emergency vehicle not detected

If the vehicle is no longer detected, it means it has passed the signal.

8. Continue the traffic light signal to green

The system continues the green signal for a short time to ensure complete passage before resuming normal operation.

9. Emergency vehicle not detected

If the YOLO model doesn't find any emergency vehicle in the initial image, the system decides that no priority is needed.

10. Continue with the operation of automatic traffic signaling system based on traffic

The system switches back to standard automatic signal control, adjusting lights based on real-time traffic flow instead of emergency detection.

Details of the components/simulation tools to be used

Details of the components/simulation tools to be used

Hardware Components

1. Arduino Uno

- Role: Serves as the central microcontroller unit for managing communication and control.

- Functionality:

  - Interfaces with the ESP32-CAM and other peripherals (e.g., LEDs, traffic light system).

  - Receives processed signals or commands from MATLAB/Simulink via serial communication (optional).

  - Can handle basic logic for traffic control and override systems.

- Connectivity:

  - Uses digital and analog I/O pins to interact with actuators and sensors.

  - Optionally interfaces with Bluetooth or Wi-Fi modules for wireless communication.

2. ESP32 Camera Module

- Role: Primary image and video capturing hardware.

- Functionality:

  - Captures live video feeds of traffic intersections.

  - Sends images or streams for processing either locally or to an external processor (e.g., MATLAB).

  - Can perform lightweight on-board image analysis using MicroPython or Arduino IDE.

- Connectivity:

  - Built-in Wi-Fi and Bluetooth support for remote streaming.

  - Can communicate with Arduino Uno via serial or I2C.

3. Image Sensors (Integrated with ESP32-CAM)

- Role: Convert optical input into digital signals (frames).

- Functionality:

  - Segment video into frames for real-time or batch processing.

  - Supports edge detection and pattern recognition (ambulance logo, shape, or flashing light).

- Processing:

  - Output can be transferred to MATLAB/Simulink for deep image processing and traffic analysis.

4. Traffic Light System (Prototype)

- Role: Mimics a real-world traffic signal for simulation and testing.

- Components:

  - LEDs for red, yellow, and green signals.

  - Relay modules or transistor switches to simulate control.

  - Powered and controlled via Arduino Uno outputs.

Software and Simulation Tools

1. MATLAB/Simulink

- Simulation Environment:

  - Models traffic junctions and signal logic using block diagrams.

  - Integrates logical flow for dynamic traffic control.

- Image and Video Processing:

  - Uses Canny Edge Detection for analyzing vehicle and ambulance movement.

  - Performs histogram analysis or frame differencing to estimate traffic density.

- Algorithm Development:

  - Logical blocks simulate decision-making:

    - Traffic light duration adjustment based on density.

    - Priority override when an ambulance is detected.

2. Logical Subsystems in Simulink

Without Ambulance (Normal Mode)

- Standard Cycle Timing:

  - Red: 5 seconds

  - Green: 5 seconds

- Function:

  - Ensures fair distribution of green time for each direction.

  - Cycle repeats based on fixed timing or minor density adjustments.

With Ambulance (Priority Mode)

- Ambulance Detection:

  - If an ambulance is identified in a particular direction:

    - Immediate override of current cycle.

    - Green light activated in ambulance’s path.

    - Maintains green until safe passage is confirmed.

- Post-Passage Recovery:

  - Returns to standard cycle after delay or confirmation.

Deliverables

1. Final Report

- Comprehensive project documentation including:

  - Problem statement.

  - Objectives and scope of the project.

  - Literature survey and references.

  - Proposed methodology with detailed explanations.

  - Simulation results and findings.

  - Conclusion and future scope.

2. Simulation Results

- MATLAB/Simulink models demonstrating:

  - Traffic light control for both normal and emergency scenarios.

  - Real-time ambulance detection and priority mechanism.

  - Traffic density estimation and its impact on signal operation.

- Screenshots or video recordings of the simulation.

3. Source Code

- Complete and well-documented source code for:

  - Ambulance detection algorithms (image/audio processing).

  - Traffic light logic implementation (in MATLAB, Python, or embedded systems code).

- Comments and explanations for reproducibility.

4. Hardware Implementation

- Functional prototype demonstrating the system's working, including:

  - Arduino setups.

  - Esp32 camera for real-time video input.

  - Traffic light simulation (using LEDs or virtual components).

- Integration of all hardware components.

5. Presentation

- Slide deck summarizing:

  - Project introduction and motivation.

  - Technical approach and components.

  - Simulation and hardware demonstration.

  - Key findings and conclusions.

- Includes visuals like block diagrams, flowcharts, and screenshots.

Timelines

Month 1: Problem Identification and Research

Week 1:

- Define the problem: Ambulance delays due to traffic congestion.

- Conduct a literature review on existing traffic management systems.

- Identify the role of IoT and AI in dynamic traffic management.

Week 2–4:

- Analyze traffic patterns and collect initial data (case studies, urban traffic models).

- Define project objectives and finalize system requirements (hardware/software).

Month 2: System Design and Initial Implementation

Week 5–6:

- Develop the methodology:

- Audio processing for ambulance siren detection.

- Image processing for traffic density analysis and vehicle detection.

- Design system architecture and block diagrams.

Week 7–8:

- Select and procure hardware: Arduino, Raspberry Pi, cameras, microphones, sensors.

- Begin implementation:

- Program initial algorithms for audio and image processing.

- Integrate sensors for data collection.

Month 3: Full Implementation and Testing

Week 9–10:

- Complete integration of hardware and software components.

- Implement traffic signal control logic based on emergency detection.

Week 11–12:

- Run simulations (e.g., MATLAB/Simulink) to validate system behavior:

- Test normal traffic scenarios.

- Test emergency vehicle prioritization.

Contribution of each member

1. Ravi Kumar (ENG21EC0095)

Responsibilities:

- Lead the problem identification phase and research literature review.

- Oversee audio processing implementation to detect ambulance sirens.

- Contribute to performance analysis and system optimization.

2. Syed Omar (ENG21EC0127)

Responsibilities:

- Design and implement image processing techniques (e.g., Canny Edge Detector).

- Develop algorithms for identifying traffic density and emergency vehicles.

- Assist with hardware integration, especially camera-related setups.

3. Uday Kiran Reddy G (ENG21EC0133)

Responsibilities:

- Manage hardware procurement and setup, including Arduino,and sensors.

- Implement signal control mechanisms for real-time traffic adjustments.

- Test system functionality during the simulation phase.

4. Mahendra B (ENG20EC0048)

Responsibilities:

- Focus on integrating IoT components and ensuring data flow between devices.

- Document the system design and assist with block diagram development.

- Lead the final presentation preparation and reporting.

Conclusion:

This smart traffic control system provides an efficient solution for reducing delays faced by emergency vehicles at traffic signals. By integrating real-time image capture and advanced object detection using the YOLO V8 model, the system can accurately identify emergency vehicles such as ambulances, fire trucks, or police cars from a distance. Upon detection, it dynamically overrides the standard traffic signaling system to prioritize the movement of these vehicles by switching the light to green.

The system further confirms the vehicle’s presence at the signal to maintain accurate control and avoid unnecessary disruptions. If the emergency vehicle is not detected at any stage, the system reverts to the normal traffic management process, ensuring seamless flow for other vehicles.

This intelligent approach not only minimizes ambulance response time but also enhances overall road safety, improves traffic efficiency, and can be integrated into existing urban smart traffic infrastructures. Its automated nature reduces the need for manual intervention, making it a scalable and reliable solution for modern cities.

Future Scope:

- Integration with GPS and IoT Devices:
Emergency vehicles can be equipped with GPS and IoT modules to directly communicate with traffic signals, further increasing detection accuracy and coordination.

- Support for Multiple Intersections:
Expanding the system to cover a network of traffic signals allows continuous priority routing for emergency vehicles across multiple junctions.

- Use of 5G Communication:
With ultra-low latency, 5G networks can enable faster real-time communication between vehicles and traffic infrastructure.

- Machine Learning-Based Traffic Prediction:
Incorporating AI models to predict traffic congestion patterns can help optimize signal timing even beyond emergency situations.

- Integration with Smart City Platforms:
The system can be a part of a larger smart city ecosystem, contributing to centralized traffic monitoring, data analytics, and emergency response planning.

- Voice and Audio Signal Detection:
Future systems could include siren sound detection to complement image processing, improving recognition even in low-visibility conditions.

- Cloud-Based Data Logging and Analytics:
Storing detection and signal data in the cloud could help analyze system performance, vehicle flow, and response times for further improvements.

- Autonomous Vehicle Compatibility:
As autonomous emergency vehicles emerge, the system could be adapted to interact with them directly, ensuring uninterrupted passage.

References

- Pandiaraj, K., et al. "RFID Based Automatic Lane Clearance for Ambulance." 2021 International Conference on Advance Computing and Innovative Technologies in Engineering (ICACITE). IEEE, 2021.

- Kamdar, Arihant, and Jigarkumar Shah. "Smart traffic system using traffic flow models." 2021 International Conference on Artificial Intelligence and Smart Systems (ICAIS). IEEE, 2021.Rachana K P, Aravind R, Ranjitha M, Spoorthi Jwanita, Soumya K, 2021, IOT Based Smart Traffic Management System, INTERNATIONAL JOURNAL OF ENGINEERING RESEARCH & TECHNOLOGY (IJERT) NCCDS – 2021 (Volume 09 – Issue 12).

- Lingani, Guy M., Danda B. Rawat, and Moses Garuba. "Smart traffic management system using deep learning for  smart city applications." 2019 IEEE 9th annual computing and communication workshop and conference (CCWC). IEEE, 2019.

- Javaid, Sabeen, et al. "Smart traffic management system using Internet of Things." 2018 20th international conference on advanced communication technology (ICACT). IEEE, 2018 S. N. Mahealani, K. B. Malakand and R. Manaker, "Device to device interaction analysis in IoT     based Smart Traffic Management System: An experimental approach," 2016 Symposium on Colossal Data Analysis and Networking (CDAN), Indore, 2016, pp. 1-6. Doi:10.1109/CDAN.2016.7570909

- Yang, Bo, Ranching Zheng, Keisuke Shimon, Tsutomu Kazakova’d Kimiko Nakano. "Evaluation of the effects of in-vehicle traffic lights on driving performances for unsignalized intersections." IET Intelligent Transport Systems11, no. 2 (2017).

- Chandramana, J., Nagarajan, R., Satheesh Kumar, K., Ajith Kumar, N., Gopinath, P. A., & Ranjith Kumar, S. (2017). Intelligent smart home automation and security system using Arduino and Wi-fi. International Journal of Engineering and Computer Science.

- IBM Bluemix (2018) Build a cloud-ready temperature sensor with the Arduino Uno and the IBM Watson IoT Platform.

- A.Balamurugan, G.Navin Siva Kumar, S. Raj Thilak, P. Sivakumar (2015) Automated emergency system in  ambulance to control traffic signals, International Journal of Engineering And Computer Science, 4(4) pp. 11533-11539.

- M. Banzi and M. Shiloh (2015) Getting Started with Arduino: The Open Source Electronics Prototyping Platform, Maker Media, Inc. (3rd edition).

Publications

Steps for Publication

Prepare a Research Paper:

- Format the paper according to the target publication's guidelines.

- Key sections to include:

- Abstract: A brief summary of the project, objectives, methodology, and results.

- Introduction: Problem statement, relevance, and objectives.

- Methodology: Detailed description of the system design and techniques used (audio/image processing, IoT, etc.).

- Results and Analysis: Simulation results, performance metrics, and discussion of findings.

- Conclusion and Future Work: Summarize the impact and propose potential enhancements.

- References: Cite all sources used in the research.

Select a Suitable Journal or Conference:

- Choose a venue relevant to IoT, AI, traffic management, or smart city solutions.

Submit Your Paper:

- Follow the submission guidelines and ensure all co-authors review the final draft.

- Include acknowledgments to your guide and institution.

Respond to Reviewer Feedback:

- Revise the paper as per the reviewer comments and resubmit if needed.

Suggested Conferences and Journals

Conferences:

- IEEE International Conference on Intelligent Transportation Systems (ITSC).

- International Conference on Artificial Intelligence and Smart Systems (ICAIS).

- International Conference on IoT and Smart Cities.

Journals:

- IEEE Transactions on Intelligent Transportation Systems.

- Springer Journal of Transportation Technologies.

- Elsevier Transportation Research Part C: Emerging Technologies.

- International Journal of Engineering Research & Technology (IJERT).